# Attachment

# Hardware Infrastructure of CAP Financial Management System Upgrade

# for The Hong Kong Council of Social Service (HKCSS)

# on behalf of 7 participating NGOs

# Hardware Infrastructure Specification

# (Tender Ref. No.: CAPFMHI-TD1)

- Existing Problems Identified

As the CAP Financial Management System was set up in 2003, and there were no major system upgrade since the system launch, most of the technologies and equipment the CAP FMS is applying have become obsolete and unreliable.

- Not compatible with latest hardware – the existing Oracle E-Business Suite is only compatible with an old version of Red Hat Enterprise Linux, Release 4.  However, the server hardware currently available in the market only supports the Red Hat Enterprise Linux starting from Release 5.  The servers of CAP FMS was replaced in 2009, and the servers have been used for 7 years, even though the servers have reached their normal lifespan, it is not feasible and flexible to replace the existing servers with new ones due to incompatibility issue.

- Existing hardware and equipment are ageing – as mentioned, the existing servers of CAP FMS have been used for 7 years, and those servers are reaching their end of life.  The server for Reporting, i.e. Oracle Discoverer, does not have sufficient capacity to accommodate the usage of multiple NGOs, and the Reporting tool is down frequently when there is high usage demand from the NGOs.  The server has to be re-booted from time to time in order to resume the Reporting tool.

- Lack of hot-backup and backup verification – the official data backup tool of Oracle Database is named RMAN.  The backup tool supports hot backup and provides verification on the backup copy after the completion of the data backup process.  The backup tool would export the data from the Oracle Database to the storage of the same server temporarily, for the process of hot-backup, and it requires at least 50% free disk space in the Database server.  Due to insufficient free disk space in the Database server, hot-backup with backup verification cannot be performed.  This causes higher risk of data backup failure or invalid data backup.

- Existing IT Facilities

The servers of the CAP FM Systems are installed in a data centre operated by a provider.  The users of the CAP FMS connect to the application through Internet with SSL encryption by a self-signed SSL certificate.

All of the service units are equipped with broadband connection and most of the PCs are installed with the operating system of Microsoft Windows 7 32bits / Windows XP for the compatibility with the CAP FMS.

There are 2 instances of Oracle E-Business Suite involved in this System Upgrade: 1 instance for multiple NGOs; and other for a single NGO.

Regarding the one for multiple NGOs, it consists of 3 server machines for Application, Database and Discoverer respectively.  The Application and Database server machines are running in the operating system of Red Hat Enterprise Linux Release 4; while the Discoverer server is running in the operating system of Windows Server 2000.

Regarding the one for single NGO, it consists of 2 server machines, one for Application and the other for Database cum Discoverer.  Both server machines are running in the operating system of Oracle Linux.  Besides, that Oracle E-Business Suite instance consists of 2 Business Groups, one for the NGO, the other one for its subsidiaries.

- Infrastructure Overview

The Hong Kong Council of Social Service is planning to build up a hardware infrastructure for supporting the Oracle E-Business Suite upgrade to R12 or above.  For the CAP FM System Upgrade, The Hong Kong Council of Social Service will upgrade the hardware infrastructure with the technologies including but not limited to:

- Server hardware

- Server virtualization (Oracle VM)

- Data backup

- Infrastructure protection

- High Availability feature of Oracle VM

- SAN Storage

VPN connection (i.e. site-to-site VPN, SSL VPN) and network settings have to be configured for the access of participating NGOs.  Please refer to the table of Appendix D to ensure the compatibility of the VPN with participating NGOs’ firewalls.

The new servers and equipment in this Project will be installed in the existing data centre and have to be connected with the existing network for facilitating the data migration.  Supplier is required to provide implementation services for the physical installation of all the server hardware and equipment.  One full rack will be cleared and reserved for the new hardware purchased in this Project.  Please fill in the table of Appendix C to indicate the arrangement of new hardware in server rack.

- Infrastructure Specification

|  | Server virtualization: |
| --- | --- |
|  | Currently, one instance of Oracle FMS consists of 3 servers currently: Application, Database and Reporting (i.e. Oracle Discoverer). After splitting the single instance of Oracle E-Business Suite into multiple, there would be 14 servers required. In order to save the server rack space, provide flexibility on server hardware assignment and replacement, and provide higher system availability (by installing multiple physical servers, and migrating the virtual machines to other spare physical servers in case one of them is failed), server virtualization would be applied in this project. A robust server hardware infrastructure would be established for hosting the multiple Virtual Machines (VMs) of 7 NGOs, which mainly consists of 4 physical servers running on a common Storage Area Network (SAN). As the 4 physical servers and the SAN needs to support the loading of total 14 virtual machines and 7 FMS (including Production and User Acceptance Test environments), enterprise-grade servers and SAN would be applied, otherwise, there might be performance issue especially during the month-end and year-end peak period of financial management. Oracle VM would be applied as it is the virtualization software certified by Oracle for the running of Oracle E-Business Suite, and it is also more favorable on the Oracle licensing scheme for Processor-based licenses. |
|  | The hardware, data backup (for Disk-to-Disk, and Disk-to-Tape, VM image and file level backup) and network equipment will be covered in this tender. The third-party Oracle Solution Vendor will be responsible for the setup of Oracle VM with High Availability, however, the Supplier of this tender has to handle the corresponding setup in the hardware and network level, e.g. the zoning of SAN storage, for supporting the server virtualization (i.e. Oracle VM). The Supplier needs to collaborate with third-party Oracle Solution Vendor for the setup of the Oracle E-Business Suite system environment, by providing advice and guidance. |
|  | For details of the preliminary plan regarding the server virtualization, please refer to Appendix A and Appendix B for details. |
|  | 7 independent Oracle E-Business Suite instances would be installed for the participating NGOs, the customizations specific to an NGO would not affect other NGOs in the “multiple instances” model. As each of the 7 Oracle E-Business Suites only contains the data of the corresponding NGO, and not identical, the users of NGOs cannot perform a testing in a common Oracle instance and then cloning the tested instance into multiple instances, as the NGOs are not supposed to view the data of other NGOs. Therefore, multiple Oracle E-Business Suite instances would be created for the NGOs to perform testing, in case there are any common technical issues, the fixing of those technical issues have to be performed multiple times. The Supplier has to provide an appropriate plan for user acceptance test and system rollout by making use of the available server resources. |
|  | Site-to-Site VPN and Remote-Access SSL VPN |
|  | Site-to-Site VPN would be formed between the data centre and the accounting head office of every NGO; and Remote-Access SSL VPN, with 2-Factors-Authentication (2FA), would be granted to the Service Unit (SU) users. The purchased licenses of Remote-Access SSL VPN can support 450 concurrent accesses. 2 firewalls would be installed for handling 2 different types of VPN connection (i.e. Site-to-Site VPN and Remote-Access SSL VPN) in order to share the loading of encryption/ decryption. For the 2FA of Remote-Access SSL VPN, the second password will be sent to the corresponding user by e-mail. Please refer to the table of Appendix D to ensure the compatibility of the VPN with participating NGOs’ firewalls. |
|  | Data backup solution |
|  | The official data backup tool of Oracle Database namely RMAN would be applied, and 50% free disk space would be reserved for the temporary space of data backup process. The setup of RMAN will be handled by the third-party Oracle Solution Vendor. Besides, Disk-to-Disk data backup would be performed by making use of the lower-speed disk pool in the SAN storage. Afterwards, Disk-to-Tape data backup would be performed by transferring the backup data into the backup tapes by backup tape library. Backup tapes would be set off-line from time to time in order to provide an off-line backup, which can protect the backup data from the malicious attacks, such as Ransomware. The Supplier is responsible for the setup of Disk-to-Disk and Disk-to-Tape data backup. |
|  | System Administration Training and Skills Transfer |
|  | At least 1 round of hands-on system administration training has to be provided for ITRC after the completion of hardware installation, covering all the necessary topics. Each session of training should be able to accommodate the class size of 10 candidates. The main objective of the system administration training is to enable the system administrators to perform daily housekeeping and maintenance works on the new Infrastructure. |
|  | During the hardware configuration period, the Supplier also needs to provide side-by-side support to ITRC to facilitate the staff to pick up the skill of tackling the technical problems. |
|  | Regarding the technical trainings stated above, 1 round of training may consists of multiple sessions and days. The Suppliers have to propose the training plan and advise the appropriate duration for each type of technical training. |
|  | High speed server-side network |
|  | A pair of Gigabit switches with resilience would be installed in the data centre, and all the equipment, including firewalls, servers, SAN and backup tape library, are interconnected via high speed Local Area Network (LAN) channels. Also, a pair of Fiber switches with resilience would be installed between SAN Storage and 4 physical servers. The fiber network would be used for data transfer of the application, data exchange between the central SAN and the 4 physical servers (for operating the VMs). A backup management server would handle the data backup process between the SAN and the backup tape library. |
|  | Server and Network Performance tuning |
|  | If there is any unreasonably poor performance in the network and server, the Supplier is responsible for studying the bottleneck and root cause of the poor performance in the new Infrastructure, and rectify the issue by configuration and performance tuning. |

- System Requirements

The System Hardware solution proposed by the Suppliers must comply with the following system requirements:

- Comply with the software and hardware infrastructure designed in Section 6.  Counter proposal is accepted if the Supplier finds any concerns or issues on the infrastructure stated in Section 6 after its fair assessment.

- System is able to perform the Hot Backup with Oracle Recovery Manager (RMAN).

- Security Risk Assessment and Audit (SRAA) has to be conducted by a third-party company with positive results after the System Upgrade of Oracle E-Business Suite (~1.5-year project).  The Supplier has the responsibility to rectify the problems and issues raised by the SRAA consulting company within the System Upgrade project timeframe.

- Software and Hardware Requirement

A preliminary design of the infrastructure is stated below for your reference:

- Hardware

| Item and Description | Quantity |
| --- | --- |
| Oracle VM Management Server: Server, 1x 2623v4 processor, 32GB RAM, 2x 15K 300GB Hard-disk, 7 x 24 Hardware Maintenance - 1x E5-2623v4 CPU - 2x 16GB 1Rx4 PC4-2400T-R Memory - 2x 300GB 12G SAS 15K 2.5in SC ENT HDD - 2x 1.83m 10A C13 UK Power Cord - 1x Smart Array P440ar/2G FIO Controller - 1x Easy Install Rail Kit - 2x 8Gb 2-port PCI-e Fiber Channel Host Bus Adapter - 2x 500W FS Plat Hot Plug Power Supply Kit - 1x 24x7 Maintenance Service Remarks: - The Oracle VM Management Server would manage the assignment of Virtual Machines to the 4 physical servers. | 1 |
| Servers for running the Oracle Virtual Machines: Server, 2x l, 192GB RAM, 2x 300GB Hard-disk, 7 x 24 Hardware Maintenance - 1x E5-2640v4 CPU - 6x 32GB 2Rx4 PC4-2400T-R Kit Memory - 2x 300GB 12G SAS 10K 2.5in SC ENT HDD - 2x 1.83m 10A C13 UK Power Cord - 1x Smart Array P440ar/2G FIO Controller - 1x 1Gb Ethernet 4P 331FLR Adapter - 1x Easy Install Rail Kit - 2x 8Gb 2-port PCI-e Fiber Channel Host Bus Adapter - 2x 800W FS Plat Hot Plug Power Supply Kit - 1x 24x7 Maintenance Service Remarks: - Oracle R12 Application requires 2 VM to support. - 4 Physical Servers will totally serve 7 NGOs and each Physical Server will run 4 Virtual Machines, and the resources consumption is very demanding. Therefore, enterprise-grade servers will be applied for ensuring the system performance. - In case one of the servers is down, the other 3 Physical Servers will take up the loading, and some spare resources are reserved so that 1 Physical Server can handle maximum 5-6 Virtual Machines when there is any incident. | 4 |
| Backup Server: Server, 1x 2623v4 processor, 32GB RAM, 2x 15K 300GB Hard-disk, 7 x 24 Hardware Maintenance - 1x E5-2623v4 CPU - 2x 16GB 1Rx4 PC4-2400T-R Kit Memory - 2x 300GB 12G SAS 15K 2.5in SC ENT HDD - 2x 1.83m 10A C13 UK Power Cord - 1x Smart Array P440ar/2G FIO Controller - 1x Easy Install Rail Kit - 1x 8Gb 2-port PCI-e Fiber Channel Host Bus Adapter - 2x 500W FS Plat Hot Plug Power Supply Kit - 1x 24x7 Maintenance Service | 1 |
| Storage for Virtual Machine data and Disk-to-Disk backup: 5x 6TB 7.2K rpm & 30x 1.8TB 10K rpm Hard-disk - 1x Product Unified (Fiber Channel /iSCSI) - 5x Universal rail kit - 1x Drive Box (LFF) - 3x Drive Box (SFF) - 2x Host I/O Module Fiber Channel 8Gbps 4port - 1x Documentation Library - 3x LAN Crossover Cable 14' - 6x Dummy Drive for SFF (2U) Trays - 2x Dummy Drive for CBSL/DBL - 1x Power Cable 250VAC 10A IEC320-C14 - 10x Power Cable - 208/220V, 1m (3ft) - 5x 6TB 7.2K rpm LFF Disk Drive - 30x 1.8TB 10K rpm SFF Disk Drive Remarks: - The SAN Storage consists of the hard-disks with 2 different read-write speeds. The faster hard-disks (i.e. 30x 1.8TB, 10K rpm) will be used for storing the Virtual Machine data, while the slower hard-disks (i.e. 5x 6TB, 7.2K rpm) will be used for Disk-to-Disk backup. - As the CAP FMS has stored the financial data of ~13 years, the data volume is huge. The current disk space usage of Application server is ~1TB and Database server is ~2TB, which includes all subscribing NGOs’ data. - RAID6 will be formed in the hard-disks for Production use (30 x 1.8TB) and the usable disk space will be reduced by ~3.6TB, equivalent to 2 hard-disks. 1 hard-disk will be reserved for hot-spare, and the spare hard-disk will take up the duty instantly when one of the hard-disks in the RAID group is failed. Therefore, total usage disk space will be 50TB. - At least 50% disk space of Database server has to be reserved for the operation of RMAN. - Spare disk space has to be reserved for 8-10 years data growth. ~100% data growth is estimated. - The 7 NGOs will use the central SAN Storage for running their Systems. Therefore, enterprise-grade SAN Storage with high read-write speed, reliability and availability will be applied in order to ensure the system performance and availability. | 1 |
| Fiber Switches for connecting 4 servers for running the Oracle Virtual Machines with the SAN storage. For resilience purpose, cross network connection is applied to avoid single point failure 24 x 8Gbps Fiber Channel port, per switch - 24P, 8Gb SWL SFPs, port side exhaust, rack kit 2 - 50/125 LC/LC PLN 50M 2f round SB 10gig OM3 48 - It acts as the backbone of the whole system and most of the communication between devices is depending on it. Therefore, enterprise-grade Fiber Switches will be applied for ensuring the system performance. | 2 |
| Gigabit Switch for connecting all the devices in the infrastructure, including firewalls, Oracle VM Management Server, the 4 physical servers for running VM, Backup Server and Tape Library. 48 ports Gigabit switch - 48-port Gigabit web-managed switch - 48 autosensing 10/100/1000BASE-T ports | 2 |
| Backup Tape Library: Tape Library, 7 x 24 Hardware Maintenance - 7 x 24 Hardware Maintenance - Drive Tape Library - LTO-7 Fiber Channel Drive Kit - 24x7 Maintenance Service | 1 |
| Firewalls for Site-to-Site VPN and Remote-Access SSL VPN: Firewall, Enterprise Protect - Firewall throughput: 45 Gbps - VPN throughput: 5.5 Gbps - IPS throughput: 10 Gbps - Ethernet: 8 GE copper + 3 FleXi Port Remarks: - At least 8 Site-to-Site VPN tunnels (for 7 NGOs + HKCSS) will be formed and ~650 users will access the Systems via Site-to-Site VPN or Remote-Access SSL VPN. - Therefore, the firewalls must be powerful enough to handle the traffic and also the data encryption. - 2 firewalls are installed for different functions, one for Site-to-Site VPN and another for Remote-Access SSL VPN, in order to share the loading. In case one of the firewalls is down, the other one will be re-configured to handle both Site-to-Site VPN and Remote-Access SSL VPN traffic. Therefore, spare capacity has to be reserved for resilience - High-end firewalls will be applied for VPN | 2 |
| KVM Switch and Consoles with monitor | 1 |

- Software

| Item and Description | Quantity |
| --- | --- |
| Operating System: |  |
| Microsoft Windows Server Standard 2016, Single Open License Program, 2 Entry-Level Core Licenses Remarks: - 1 Physical Backup Server | 2 |
| Oracle Linux 7 or above Remarks: - 14 VMs for Oracle E-Business Suite + 1 Physical Server for Oracle VM Management Server - Oracle E-Business Suite R12 instances of 2 small-size NGOs share 1 Group of VMs - 1 Group of VMs consists of 2 VMs for the operation of Oracle E-Business Suite, i.e. 1 for Application + Reporting; 1 for Database - 1 Group of VMs (i.e. 2 VMs) is reserved for User Acceptance Test (UAT) use - Total: (6 Groups for Production + 1 Group for UAT) x 2 VMs = 14 VMs | 14 |
| Oracle VM Remarks: - 4 Oracle VM Servers will form Oracle VM cluster | 4 |
| Data Backup: |  |
| Backup Exec Server Edition Windows, 1 Server On-premise Standard License + Essential Maintenance Bundle Initial 12 Months Corporate Remarks: - Server for the Data Backup software | 1 |
| Backup Exec Option Library Expansion Windows, 1 Device On-premise Standard License + Essential Maintenance Bundle Initial 12 Months Corporate Remarks: - for supporting backup tape library | 1 |
| Backup Exec Agent for Linux, 1 Server On-premise Standard License + Essential Maintenance Bundle Initial 12 Months Corporate Remarks: - Agent for the Data Backup software - For 14 VMs running on Linux - For file-level data backup from Linux to disk or to tape | 14 |
| Other Data Backup solution for Oracle VM virtual machine level data backup |  |
| Anti-virus: |  |
| Kaspersky Endpoint Security for Business – Select Remarks: - For 14 Linux platforms and 2 Windows platforms - Linux platform: 14 VMs of Oracle E-Business Suite + 1 Physical Server of Oracle VM Management Server - Windows platform: 1 Physical Server of Backup Server | 16 |

Suppliers would need to make a fair assessment on those planned hardware and software configuration described above to ensure that the proposed solution/software product is compatible and with sufficient capacity.  Besides, Suppliers would also need to ensure that such hardware/software configuration supports proper data backup (even for hot backup) and security control.

Supplier has to make an appropriate plan for the hardware resources allocation among the participating NGOs, including CPU, RAM, storage, etc,.

Suppliers need to propose a Disaster Recovery (DR) Plan and conduct a DR drill during the implementation period upon the request of The Hong Kong Council of Social Service or 7 participating NGOs.

Suppliers should counter-propose the hardware/software configuration required if they find that there is deficiency in the above-stated hardware, software and infrastructure configuration.

Server hardware, general software (such as operating system, anti-virus, backup software, etc.) and the infrastructure setup will be handled in this tender.  However, the Oracle related software and its installation, including but not limited to Oracle VM, Oracle Linux, Oracle RMAN, Oracle Application and Database, will be handled by the third-party Oracle Solution Vendor.  The physical server machines and the SAN storage, with proper physical installation, will be handed over to the third-party Oracle Solution Vendor for subsequent system implementation works.  The Supplier has to collaborate with the third-party Oracle Solution Vendor and SRAA Consulting Company for setting up the entire infrastructure by providing guidance and advice.

- Schedules

  - Compliance

(* Please tick the appropriate.)

[  ]	I/We confirm that our proposal comply fully with the required specifications, described in Sections 1 to 7 of this Attachment.

[  ]	I/We confirm that our proposal does not comply fully with the required specifications, described in Sections 1 to 7 of this Attachment, in the following aspects:

  - Risk Management

Please state your proposed approach for risk management including but not limited to the following aspects:

- Transaction Data Backup

- System Backup:

- Resilience:

- Security:

- Contingency:

- Other risk factors:

  - System Administration Training and Skills Transfer

Please describe your proposed approach and plan for all technical training, including system walkthrough, UAT briefing, administration training, skills transfer, etc.  Please provide the number of sessions and hours to be provided.

  - Documentation and Deliverables

Please state if the following documentation/deliverables will be provided:

| Documentation/ Deliverables | Comply (Yes/ No) |
| --- | --- |
| Project Initiation Document |  |
| Hardware installation and infrastructure setup |  |
| Data Backup Solution setup |  |
| VPN Connection Setup |  |
| Disaster Recovery Plan and Procedures |  |
| Disaster Recovery Drill |  |
| System Administration Manual |  |
| System Administration Training/ Skill Transfer |  |
| System Nursing |  |
| Vulnerability Fixing for SRAA |  |
| Free Warranty |  |

Please state the documentation/deliverables other those listed above:

  - Project Schedule

Please provide the project schedule by Month Number, e.g. Month 1 is the 1st month since the formal project commencement.

| ## # | ## Major task/ milestone | ## From ## (Month No.) | ## To ## (Month No.) |
| --- | --- | --- | --- |
|  | ## Project Initiation |  |  |
|  | ## Hardware and Equipment Delivery |  |  |
|  | ## Infrastructure Setup |  |  |
|  | ## VPN Connection Setup |  |  |
|  | ## Backup Solution Setup |  |  |
|  | ## System Administration Training/ Skill Transfer |  |  |
|  | ## System Administration Manual |  |  |
|  | ## User Acceptance Test |  |  |
|  | ## System Live Run |  |  |
|  | ## System Nursing |  |  |
|  | ## Vulnerability Fixing for Security Risk Assessment and Audit |  |  |

Remarks: Task #1 – 3 are expected to be completed within 2 months after the project commences.  We have to match the progress of Oracle Application implementation to setup the VPN connection between 7 NGOs one by one (Task #4), the remaining project task #5 – 10 will be conducted after the Oracle E-Business Suite System Upgrade (estimated as 1.5 year later).  Task #11 will depends on the schedule of Security Risk Assessment and Audit (SRAA) exercise (estimated as 1.5 year later).

  - Quotation Breakdown

For quotation breakdown details, the supplier should make a fair assessment to make sure that the products and services quoted fulfill the tender specification.

|  | Part Number | ITEM | Unit of Measure | Qty. | Unit Price | Total |
| --- | --- | --- | --- | --- | --- | --- |

  - Payment Schedule

| Milestone | Percentage of Contract Sum |
| --- | --- |
| Initial Payment | 30 |
| Hardware and Equipment Delivery | 30 |
| VPN Connection (completed all NGOs) | 10 |
| User Acceptance Test(completed all NGOs) | 10 |
| System Live Run (after SRAA) (completed all NGOs) | 10 |
| Project Completion | 10 |

Note:  Suppliers have to offer at least 90 days payment term for the issued invoices.

(* Please tick the appropriate.)

[  ]	I/We confirm that our proposal comply fully with the payment schedule described above.

[  ]	I/We confirm that our proposal does not comply fully with the Payment schedule described above in the following aspects:

  - System Support and Maintenance

Please describe the scope of services and service level for the system support and maintenance in the following aspects:

- Helpdesk services:

(Service hours, Response time of calls should be specified.)

- Offers regarding out-scope services:

- On-site support services:

  - Free Warranty Period and Service Coverage

The duration of free warranty period upon the completion of the project, i.e. end of system nursing, will be             calendar months (not less than 90 calendar months).

  - Resume of Major Project Team Members

Please supply the resumes of the major project team members.

  - Reference Projects

Please list your reference projects in the table below and provide supplementary details if relevant.

| # | System/ Project Name | Client | Supplementary |
| --- | --- | --- | --- |

  - Validity of Offer

The valid period of the Proposal is start counting after Tender Closing date for                  calendar days (not less than 90 calendar days).

- The Allocation of Virtual Machines

| NGO Number (Referring to the network diagram) | Number of VMs Assigned |
| --- | --- |
| NGO-1 | 2 |
| NGO-2 | 2 |
| NGO-3 | 2 |
| NGO-4 | 2 |
| NGO-5 | 2 |
| NGO-6 & NGO-7 NGO-7 and NGO-8 are presenting 2 small-size NGOs. For the consideration of cost effectiveness, the Oracle E-Business Suite instances of those 2 NGOs would share a single Group of VMs. | 2 |
| User Acceptance Test 1 Group of VMs would host 2 Oracle E-Business Suite instances simultaneously. | 2 |

- Network Diagram of Planned Infrastructure

- Server equipment allocation in server rack

# Server Rack

| # Rack Cabinet Location | # Item Name | # Remarks |
| --- | --- | --- |
| # 42 |  |  |
| # 41 |  |  |
| # 40 |  |  |
| # 39 |  |  |
| # 38 |  |  |
| # 37 |  |  |
| # 36 |  |  |
| # 35 |  |  |
| # 34 |  |  |
| # 33 |  |  |
| # 32 |  |  |
| # 31 |  |  |
| # 30 |  |  |
| # 29 |  |  |
| # 28 |  |  |
| # 27 |  |  |
| # 26 |  |  |
| # 25 |  |  |
| # 24 |  |  |
| # 23 |  |  |
| # 22 |  |  |
| # 21 |  |  |
| # 20 |  |  |
| # 19 |  |  |
| # 18 |  |  |
| # 17 |  |  |
| # 16 |  |  |
| # 15 |  |  |
| # 14 |  |  |
| # 13 |  |  |
| # 12 |  |  |
| # 11 |  |  |
| # 10 |  |  |
| # 9 |  |  |
| # 8 |  |  |
| # 7 |  |  |
| # 6 |  |  |
| # 5 |  |  |
| # 4 |  |  |
| # 3 |  |  |
| # 2 |  |  |
| # 1 |  |  |

- Firewall models of participating NGOs for VPN connection

| No. | Firewall Model for VPN connection |
| --- | --- |
|  | FortiGate FortiWifi60C |
|  | Cisco RV320-K9-G5 |
|  | Juniper SSG 5 |
|  | Check Point 5400 |
|  | Check Point 5600 |
|  | Palo Alto Networks PA-3020 |
|  | Juniper SSG140 |
|  | Fortinet 300D |